SYSTEM LOGIC – SMART ROOM ENVIRONMENTAL MONITORING & ALERT SYSTEM

What This System Does?

The Arduino continuously monitors temperature and light level.

It makes two independent decisions every cycle:

1. Room light → ON if dark, OFF if bright.

2. Alert LED → BLINK if too hot, OFF if normal.

All sensor values and current states are printed to the Serial Monitor.

THE LOGIC FLOW (STEP BY STEP)

START LOOP

► Read light sensor (A0) → get value (0-1023)

► Read temperature sensor (A1) → convert to °C

► Print: "Temp: X°C | Light: Y"

► IF light value < 400 (dark)

THEN turn room light ON, print "[ON] LIGHTS"

ELSE turn room light OFF

► IF temperature > 30°C (hot)

THEN print "[!] OVERHEAT"

turn alert LED ON, wait 200ms

turn alert LED OFF, wait 200ms

ELSE turn alert LED OFF, wait 400ms

► Print newline (move to next line)

► REPEAT forever

Key Logic Decisions Explained

1. Light Logic – Steady ON/OFF

| Condition | Action | Reasoning |
| --- | --- | --- |
| lightVal < 400 | Room LED = ON steady | Darkness detected → need light. |
| lightVal >= 400 | Room LED = ON steady | no light needed. |

Logic type: Simple threshold comparison with binary output.

No delays – responds instantly.

- Temperature Logic – Blinking Alert

| Condition | Action | Reasoning |
| --- | --- | --- |
| tempC > 30.0 | Alert LED blinks (200ms ON / 200ms OFF) + print "OVERHEAT" | Normal temp – pace serial output. |
| tempC <= 30.0 | Alert LED OFF + 400ms delay | Blinking catches attention |

Why blinking? A steady LED might be ignored. Blinking is a standard alert pattern.

Why a delay when normal? Without the 400ms delay, the loop would run thousands of

times per second, filling the Serial Monitor with unreadable data.